Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section 28 21 00: Video Surveillance

Section 28 21 11: Analog Cameras

Splicing Controller

- – GENERAL

  - SUMMARY

    - Section Includes

      - Section 28 21 17: Video Surveillance – Surveillance Cameras – Camera Housings

      - Section 28 21 19: Video Surveillance – Surveillance Cameras – Camera Mounts

      - Section 28 27 00: Video Surveillance – Video Surveillance Sensors

    - Related Sections

      - [Section 28 33 15: Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

********** Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

| STANDARD | NA |
| --- | --- |

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - List certifying bodies (CE, FCC, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - INSTALLATION REQUIREMENTS

    - Operating Temperature And Humidity

    - Power Requirements

  - WARRANTY

    - Provide manufacturer’s warranty covering [2] years for replacement and repair of defective equipment. Warranty varies country to country.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

ZHEJIANG DAHUA VISION TECHNOLOGY CO., LTD.

Address: No.1399, Binxing Road, Binjiang District, Hangzhou, P.R. China

Postcode: 310053

Tel: +86-571-87688888 28933188

Email: dhoverseas@dhvisiontech.com

Website: www.dahuasecurity.com

    - Substitutions:

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - Splicing Controller | DHI-DSCON8000-S13

| System | Main Processor | 64 High-performance multi-core Processor |
| --- | --- | --- |
| System | Operating System | Embedded Linux operating system |
| System | Slot | Includes a total of 15 slots. Each of the following items has its own slot: the switching card and main control card. The remaining 13 slots are for input and output cards: 2 for input, and 6 for output. The remaining 5 slots can be used for both input and output |
| System | Cabinet | Included: Single power supply, single main control card and intelligent temperature control fan; Optional: Dual power supplies and dual main control cards |
| System | Touch Screen | Equipped with an 8-inch touch screen with the resolution of 1280 × 800 that facilitates operations |
| Port | USB | 1 × USB 3.0 port |
| Port | Network Port | 2 × Gigabit ethernet ports and 2 × gigabit fiber ports |
| Port | RS-232 | 2 |
| Port | RS-485 | 2 |
| Port | HDMI | 1 |
| Port | Alarm Input Port | 1 × alarm input port (reserved) |
| Port | Alarm Output Port | 1 × alarm output port (reserved) |
| Port | Audio Input Port | 1 × 3.5 mm jack port (reserved) |
| Port | Audio Output Port | 1 × 3.5 mm jack port and 1 set of spring-loaded terminal blocks |
| Port | Type-C Port | 1 × Type-C port |
| VI0401HH-DC8000 | Video Input | 4-channel HDMI 1.4 port |
| VI0401HH-DC8000 | Audio Input | 4-channel HDMI embedded audio port |
| VI0401HH-DC8000 | Video Input Resolution | Acquires and inputs signals at the following resolutions: 1024 × 768@60 Hz, 1280 × 720@60 Hz, 1280 × 1024@60 Hz, 1366 × 768@60 Hz, 1440 × 900@60 Hz, 1600 × 1200@60 Hz, 1920 × 1080@60 Hz, 1920 × 1200@60 Hz, 3840 × 2160@30 Hz, and 4096 × 2160@30 Hz; You can customize resolutions; Width range: 800–4,094 pixels; Height range: 600–4,094 pixels; The maximum resolution at the width of 4,094 pixels is 4094 × 1140@60 Hz, while at the maximum height of 4,094 pixels, the resolution is 1024 × 4094 @60 Hz |
| VI0401HH-DC8000 | HDCP | HDCP 1.4 is supported |
| VI0401HH-DC8000 | Occupied Slot | Occupies 1 input slot |
| VI0801HH-DC8000 | Video Input | 8-channel HDMI 1.4 port |
| VI0801HH-DC8000 | Audio Input | 8-channel HDMI embedded audio port |
| VI0801HH-DC8000 | Video Input Resolution | Acquires and inputs signals at the following resolutions: 1024 × 768@60 Hz, 1280 × 720@60 Hz, 1280 × 1024@60 Hz, 1440 × 900@60 Hz, 1600 × 1200@60 Hz, 1920 × 1080@60 Hz, 1920 × 1200@60 Hz, 3840 × 2160@30 Hz, and 4096 × 2160@30 Hz; You can customize resolutions; Width range: 800–4,094 pixels; Height range: 600–4,094 pixels; The maximum resolution at the width of 4,094 pixels is 4094 × 1140@60 Hz, while at the maximum height of 4,094 pixels, the resolution is 1024 × 4094 @60 Hz |
| VI0801HH-DC8000 | HDCP | HDCP 1.4 is supported |
| VI0801HH-DC8000 | Occupied Slot | Occupies 1 input slot. Performance is halved when installed in slot 13 |
| VI0201EHP-DC8000 | Video Input | 2-channel HDMI 2.0 port and 2-channel DP 1.2 port. You can select either one for the input channel |
| VI0201EHP-DC8000 | Audio Input | Supports 2-channel embedded audio input through the HDMI port or DP port |
| VI0201EHP-DC8000 | Video Input Resolution | Acquires and inputs signals at the following resolutions: 1024 × 768@60 Hz, 1280 × 720@60 Hz, 1280 × 1024@60 Hz, 1366 × 768@60 Hz, 1440 × 900@60 Hz , 1600 × 1200@60 Hz, 1920 × 1080@60 Hz, 1920 × 1200@60 Hz, 1920 × 2160@60 Hz, 3840 × 1080@60 Hz, and 3840 ×2160@60 Hz; You can customize resolutions; Width range: 800–8,192 pixels; Height range: 600–8,192 pixels; The maximum resolution at the width of 8,192 pixels is 8192 × 1140@60 Hz, while at the maximum height of 8,192 pixels, the resolution is 1024 × 8192@60 Hz |
| VI0201EHP-DC8000 | HDCP | HDCP 2.2 is supported |
| VI0201EHP-DC8000 | Occupied Slot | Occupies 1 input slot |
| VI0401EHP-DC8000 | Video Input | 4-channel HDMI 2.0 port and 4-channel DP 1.2 port. You can select either one for the input channel |
| VI0401EHP-DC8000 | Audio Input | Supports 4-channel embedded audio input through the HDMI port or DP port |
| VI0401EHP-DC8000 | Video Input Resolution | Acquires and inputs signals at the following resolutions: 1024 × 768@60 Hz, 1280 × 720@60 Hz, 1280 × 1024@60 Hz, 1440 × 900@60 Hz, 1600 × 1200@60 Hz, 1920 × 1080@60 Hz, 1920 × 1200@60 Hz, 1920 × 2160@60 Hz, 3840 × 1080@60 Hz, and 3840 ×2160@60 Hz; You can customize resolutions; Width range: 800–8,192 pixels; Height range: 600–8,192 pixels; The maximum resolution at the width of 8,192 pixels is 8192 × 1140@60 Hz, while at the maximum height of 8,192 pixels, the resolution is 1024 × 8192@60 Hz |
| VI0401EHP-DC8000 | HDCP | HDCP 2.2 is supported |
| VI0401EHP-DC8000 | Occupied Slot | Occupies 1 input slot. Performance is halved when installed in slot 13 |
| DEC-DC8000 | Video Decoding Resolution | Supports video decoding at 32 MP, 16 MP, 12 MP, 8 MP, 5 MP, 3 MP, UXGA, 1080p,720p, D1, HD1, 2CIF, CIF, and QCIF. (32 MP, 16 MP, and 12 MP supported only with H.265) |
| DEC-DC8000 | Video Format | Supports decoding of video compression formats: H.265, H.264, MJPEG, MPEG4, SVAC, and MPEG2 |
| DEC-DC8000 | Decoding Capability | Single-card decoding capability: 2 channels@32 MP (25 fps), 10 channels@8 MP (25 fps), 14 channels@6 MP (25 fps), 18 channels@5 MP (25 fps), 28 channels@3 MP (25 fps), and 36 channels@1080p (30 fps) (maximum 36 decoding channels; 32 MP supported only with H.265) |
| DEC-DC8000 | Occupied Slot | Occupies 1 input slot. Available slots: 1, 3, 5, 7, 9, and 11. A unit supports a maximum of 2 decoding extension cards. If more than 2 cards are installed, they will work in ascending order of slot numbers |
| VS20N-DC8000 | Video Output | 20-channel RJ-45 port |
| VS20N-DC8000 | Load Capacity | A single card can transmit images with up to 13 million pixels while a single network port can transmit images with up to 650,000 pixels |
| VS20N-DC8000 | Occupied Slot | Occupies 2 output slots |
| VS20N-DC8000NW | Video Output | 20-channel RJ-45 port |
| VS20N-DC8000NW | Load Capacity | A single card can transmit images with up to 13 million pixels while a single network port can transmit images with up to 650,000 pixels |
| VS20N-DC8000NW | Occupied Slot | Occupies 2 output slots |
| VO0801HH-DC8000 | Video Output | 8-channel HDMI 1.3 port |
| VO0801HH-DC8000 | Video Output Resolution | Supports the following output resolutions: 1920 × 1200@60 Hz, 1920 × 1080@60 Hz, 1600 × 1200@60 Hz, 1400 × 1050@60 Hz, 1600 × 900@60 Hz, 1366 × 768@60 Hz, 1280 × 1024@60 Hz, 1280 × 720@60 Hz and 1024 × 768@60 Hz; You can customize resolutions; Width range: 200–4,096 pixels; Height range: 200–4,096 pixels; The maximum resolution at the width of 8,192 pixels is 4096 × 480@60 Hz, while at the maximum height of 8,192 pixels, the resolution is 480 × 4096@60 Hz |
| VO0801HH-DC8000 | Occupied Slot | Occupies 1 output slot |
| VO0201EH-DC8000 | Video Output | 2-channel HDMI 2.0 port |
| VO0201EH-DC8000 | Video Output Resolution | Supports the following output resolutions: 4096 × 2160@60 Hz, 3840 × 2160@60 Hz, 1920 × 1200@60 Hz, 1920 × 1080@60 Hz, 1280 × 1024@60 Hz, 1280 × 720@60 Hz, 1024 × 768@60 Hz; You can customize resolutions; Width range: 200–8,192 pixels; Height range: 200–8,192 pixels; The maximum resolution at the width of 8,192 pixels is 8192 × 1000@60 Hz, while at the maximum height of 8,192 pixels, the resolution is 1000 × 8192@60 Hz |
| VO0201EH-DC8000 | Occupied Slot | Occupies 1 output slot |
| PRE-DC8000 | Live Video Display Performance | Supports previewing acquired signals up to 32 channels or 4-channel video wall display |
| PRE-DC8000 | Resolution | Acquired signal preview resolution: 704 × 576@25 Hz; Video wall display resolution: 1920 × 1080@25 Hz |
| PRE-DC8000 | Occupied Slot | Occupies 1 output slot |
| Function | Max System Configuration | You can insert a total of up to 13 cards: 2 for input, 6 for output, and the remaining 5 can be used for either input or output cards; You can insert up to 7 input cards that support the following resolutions: 56-channel 1080p@60 Hz, or 26-channels 4K@60 Hz; You can insert up to 11 output cards that support the following resolutions: 88-channel 1080p@60 Hz, or 22-channel 4K@60 Hz; For sending cards, you can insert up to 5 cards with a total of 100 network ports to transmit images with up to 65 million pixels |
| Function | Layer Capability | Supports up to 56 layers in 2K resolution or 14 layers in 4K resolution |
| Function | Spliced Screen Function | Splicing and zoom, video fusion, roaming, windowing, and OSD |
| Function | Equipment Management | Supports custom business trees with different structures. Supports adjusting the order of nodes such as channels |
| Function | SDK | Supports secondary development, and provides platform SDK |
| Function | Control Mode | The device supports external monitors and mice, features its own operation page, and also supports PC clients or network keyboards |
| Function | Multi-screen Display | Supports 1/4/6/8/9/16/25/36 screen splits and supports M × N custom splits |
| General | Power Supply | 100–240 VAC, 50–60 Hz，8 A，hot swapping |
| General | Power Redundancy | Dual (optional) |
| General | Power Consumption | ≤ 550 W |
| General | Operating Temperature | –10 °C to +50 °C (+14 °F to +122 °F) |
| General | Operating Humidity | 10%–90% (RH), non-condensing |
| General | Storage Temperature | –10 °C to +50 °C (+14 °F to +122 °F) |
| General | Storage Humidity | 10%–90% (RH), non-condensing |
| General | Product Dimensions | 221.8 mm × 484.0 mm × 416.0 mm (8.73" × 19.06" × 16.38") (H × W × D) |
| General | Packaging Dimensions | 443 mm × 645 mm × 585 mm (17.44" × 25.39" × 23.03") (H × W × D) |
| General | Gross Weight | 20.6 kg (45.42 lb) |
| General | Net Weight | 14.93 kg (32.92 lb) |
| General | Series | DSCON8000 |
| General | Type | Server |

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION