Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section 28 21 00: Video Surveillance

Section 28 21 11: Analog Cameras

Distributed Ultra HD Encoder/Decoder

- – GENERAL

  - SUMMARY

    - Section Includes

      - Section 28 21 17: Video Surveillance – Surveillance Cameras – Camera Housings

      - Section 28 21 19: Video Surveillance – Surveillance Cameras – Camera Mounts

      - Section 28 27 00: Video Surveillance – Video Surveillance Sensors

    - Related Sections

      - [Section 28 33 15: Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

********** Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

| STANDARD | NA |
| --- | --- |

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - List certifying bodies (CE, FCC, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - INSTALLATION REQUIREMENTS

    - Operating Temperature And Humidity

    - Power Requirements

  - WARRANTY

    - Provide manufacturer’s warranty covering [2] years for replacement and repair of defective equipment. Warranty varies country to country.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

ZHEJIANG DAHUA VISION TECHNOLOGY CO., LTD.

Address: No.1399, Binxing Road, Binjiang District, Hangzhou, P.R. China

Postcode: 310053

Tel: +86-571-87688888 28933188

Email: dhoverseas@dhvisiontech.com

Website: www.dahuasecurity.com

    - Substitutions:

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - Distributed Ultra HD Encoder/Decoder | DHI-DSMS0115EPI-4K

| System | Main Processor | High-performance embedded processor |
| --- | --- | --- |
| System | Operating System | Embedded Linux |
| Function | Interoperability | RTSP; Private |
| Function | Video Compression | H.265;H.264 |
| Function | Audio Compression | PCM |
| Function | Bit Rate Type | Composite stream; video stream |
| Function | Encoding Capacity | Main stream: 4096 × 2160@60 fps; 3840 × 2160@60 fps; 2560 × 1440@60 fps; 1920 × 1200@60 fps; 1920 × 1080P@60 fps; 1600 × 1200@60 fps; 1440 × 900@60 fps; 1280 × 1024P@60 fps; 1280 × 960P@60 fps; 1280 × 720P@60 fps; Sub stream: D1: 704 × 576 (PAL); HD1: 352 × 576 (PAL); BCIF: 704 × 288 (PAL); CIF: 352 × 288 (PAL) |
| Function | Input Resolution | 4096 × 2160@60 fps; 3840 × 2160@60 fps; 2560 × 1440@60 fps; 1920 × 1200@60 fps; 1920 × 1080@60 fps; 1600 × 1200@60 fps; 1440 × 900@60 fps; 1280 × 1024@60 fps; 1280 × 960@60 fps; 1280 × 720@60 fps; 1024 × 768@60 fps |
| Port | Display Screen | 2" TFT screen |
| Port | Button | 5 × Thin film buttons (reserved); 1 × Reset button |
| Port | Video Input | 1 × HDMI 2.0; 1 × DP 1.2 |
| Port | Video Output | 1 × HDMI 2.0 loop output (HDMI IN or DP IN loop output) |
| Port | Audio Output | 1 × 3.5 mm JACK |
| Port | Audio Input | 1 × HDMI/DP embedded audio port; 2 × 3.5 mm JACK |
| Port | Two-way Audio | 1 × 3.5 mm earphone JACK (reserved); 2 × 3.5 mm JACK (reserved) |
| Port | Alarm Input | 2 (reserved) |
| Port | Alarm Output | 2 (reserved) |
| Port | Network Port | 1 × 10 M/100 M/1000 M self-adaptive RJ-45 Ethernet port; 1 × 1000 M SFP optical port; hot standby is supported |
| Port | RS-232 | 1 × DB9 (for device maintenance and debugging) |
| Port | USB | 3 × USB ports: 1 × USB (keyboard and mouse); 1 × USB (USB pass-through); 1 × USB 3.0 (reserved) |
| Port | TF Card Slot | 1 (reserved) |
| Port | RS-485 | 1 (reserved) |
| General | Power Supply | 12 VDC, 2.0 A; PoE |
| General | Power Consumption | ≤24 W |
| General | Operating Temperature | –10 °C to +45 °C (+14 °F to +113 °F) |
| General | Operating Humidity | 10%–95% (RH), non-condensing (86 kPa–106 kPa) |
| General | Product Dimensions | 44 mm × 190 mm × 130 mm (1.73" × 7.48" × 5.12") (H × W × D) |
| General | Packaging Dimensions | 93 mm × 330 mm × 188 mm (3.66" × 12.99" × 7.48") (L × W × H) |
| General | Gross Weight | 1.46 kg (3.21 lb） |
| General | Net Weight | 0.98 kg (2.16 lb） |

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION