Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section 28 21 00: Video Surveillance

Section 28 21 11: Analog Cameras

VIDEO MATRIX PLATFORM

- – GENERAL

  - SUMMARY

    - Section Includes

      - Section 28 21 17: Video Surveillance – Surveillance Cameras – Camera Housings

      - Section 28 21 19: Video Surveillance – Surveillance Cameras – Camera Mounts

      - Section 28 27 00: Video Surveillance – Video Surveillance Sensors

    - Related Sections

      - [Section 28 33 15: Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

********** Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

| STANDARD | CE; FCC |
| --- | --- |

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - List certifying bodies (CE, FCC, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - INSTALLATION REQUIREMENTS

    - Operating Temperature And Humidity

    - Power Requirements

  - WARRANTY

    - Provide manufacturer’s warranty covering [2] years for replacement and repair of defective equipment. Warranty varies country to country.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

ZHEJIANG DAHUA VISION TECHNOLOGY CO., LTD.

Address: No.1399, Binxing Road, Binjiang District, Hangzhou, P.R. China

Postcode: 310053

Tel: +86-571-87688888 28933188

Email: dhoverseas@dhvisiontech.com

Website: www.dahuasecurity.com

    - Substitutions:

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - VIDEO MATRIX PLATFORM | DHI-M60-12U-E

| System | Main Processor | 64 High-performance multi-core Processor |
| --- | --- | --- |
| System | Operating System | Embedded Linux operating system |
| System | Bus | PCI-E |
| System | Slot | 1 × main control card slot; 10 × audio and video card slots. supports the combination of all cards plugged |
| System | Cabinet | Dual power supply, main control card, PCI-E bus rear panel, interlligent temperature control fan |
| Port | USB | 3 × USB 2.0 port (1 for main control card; 2 for port panel) |
| Port | Network Port | 8 × RJ45 port, supports 1,000 M Ethernet port; 8 × SFP optical ports |
| Port | RS-232 | 1 × DB-9 port, used for system debugging or central control |
| Port | VGA | 1 |
| DH-VEC3204FB-M60 (Analog Video) | Video Input | BNC (1.0Vp-p, impedance: 75 Ω) |
| DH-VEC3204FB-M60 (Analog Video) | Audio Input | (single card) 32-ch, DB26 port (2.0Vp-p, impedance: 1 KΩ) |
| DH-VEC3204FB-M60 (Analog Video) | Signal Mode | PAL/NTSC self-adaptive |
| DH-VEC3204FB-M60 (Analog Video) | Encoding Format | H.264; MPEG4 |
| DH-VEC3204FB-M60 (Analog Video) | Encoding Capability | (single card) 32-ch D1 |
| DH-VEC3204FB-M60 (Analog Video) | Low Speed Data Port | 2-ch RS485, supports PTZ protocol of main stream Dome |
| DH-VEC1604HC-M60 (HDCVI) | Video Input | BNC (1.0Vp-p, impedance: 75 Ω) |
| DH-VEC1604HC-M60 (HDCVI) | Audio Input | Integrated in HDMI port |
| DH-VEC1604HC-M60 (HDCVI) | Signal Mode | HDCVI |
| DH-VEC1604HC-M60 (HDCVI) | Encoding Format | (single card) 16-ch 1080p |
| DH-VEC1604HC-M60 (HDCVI) | Encoding Capability | (single card) 16-ch, supports HDCVI standard and transmitting audio in sync with video using coaxial cables |
| DH-VEC1604HC-M60 (HDCVI) | Low Speed Data Port | 2-ch RS485, supports PTZ protocol of main stream Dome |
| DH-VEC0804US-M60 (3G-SDI HD Video) | Video Input | BNC (1.0Vp-p, impedance: 75 Ω) |
| DH-VEC0804US-M60 (3G-SDI HD Video) | Audio Input | (single card) 8-ch, DB26 port (2.0Vp-p, impedance: 1 KΩ) |
| DH-VEC0804US-M60 (3G-SDI HD Video) | Signal Mode | 3G-SDI HD video |
| DH-VEC0804US-M60 (3G-SDI HD Video) | Encoding Format | H.264; MPEG4 |
| DH-VEC0804US-M60 (3G-SDI HD Video) | Encoding Capability | (single card) 8-ch 1080p@60 fps |
| DH-VEC0804US-M60 (3G-SDI HD Video) | Low Speed Data Port | 2-ch RS485, supports PTZ protocol of main stream Dome |
| DH-VEC1604HS-M60 (HD-SDI HD Video) | Video Input | BNC (1.0Vp-p, impedance: 75 Ω) |
| DH-VEC1604HS-M60 (HD-SDI HD Video) | Audio Input | (single card) 16-ch, DB26 port (2.0Vp-p, impedance: 1 KΩ) |
| DH-VEC1604HS-M60 (HD-SDI HD Video) | Signal Mode | HD-SDI HD video |
| DH-VEC1604HS-M60 (HD-SDI HD Video) | Encoding Format | H.264; MPEG4 |
| DH-VEC1604HS-M60 (HD-SDI HD Video) | Encoding Capability | (single card) 16-ch 1080p |
| DH-VEC0804HD-M60 (DVI/VGA/HDMI HD Video) | Video Input | DVI-I port, VGA (with included converter); supports VGA, DVI and HDMI single input |
| DH-VEC0804HD-M60 (DVI/VGA/HDMI HD Video) | Audio Input | (single card) 8-ch, DB15 port (2.0Vp-p, impedance: 1 KΩ) |
| DH-VEC0804HD-M60 (DVI/VGA/HDMI HD Video) | Encoding Format | H.264; MPEG4 |
| DH-VEC0804HD-M60 (DVI/VGA/HDMI HD Video) | Encoding Capability | (single card) 8-ch 1080p Resolution: 1080p; 720p; UXGA; SXGA+; SXGA; XGA; SVGA; VGA VGA and YPbPr progressive component signal RGBHV, SOG and HDMI (with converter) |
| DH-VEC0804HD-M60 (DVI/VGA/HDMI HD Video) | Low Speed Data Port | 2-ch RS485, supports PTZ protocol of main stream Dome |
| DH-VEC0805UH-M60 (4K Encoding Card) | Video Input | 8-ch HDMI, 2-ch VGA |
| DH-VEC0805UH-M60 (4K Encoding Card) | Audio Input | (single card) 8-ch, DB15 port (2.0Vp-p, impedance: 1 KΩ) |
| DH-VEC0805UH-M60 (4K Encoding Card) | Signal Mode | HDMI; VGA; DVI (with converter) |
| DH-VEC0805UH-M60 (4K Encoding Card) | Encoding Format | H.264; H.265 |
| DH-VEC0805UH-M60 (4K Encoding Card) | Encoding Capability | (single card) max. 4-ch 4K (4096 × 2160@24 fps/3840 × 2160@30 fps + 6 × 1080p (1920 × 1080@60 fps) supports encoding 4096 × 2160; 3840 × 2160; 2560 × 1440; 1920 × 1080; 1280 × 720; 704 × 576 |
| DH-VEC0805UH-M60 (4K Encoding Card) | Low Speed Data Port | 1-ch RS485, supports PTZ protocol of main stream Dome |
| DH-VEC1605UH-M60 (4K Encoding Card) | Video Input | 16-ch HDMI, 4-ch VGA |
| DH-VEC1605UH-M60 (4K Encoding Card) | Audio Input | (single card) 16-ch, DB15 port (2.0Vp-p, impedance: 1 KΩ) |
| DH-VEC1605UH-M60 (4K Encoding Card) | Signal Mode | HDMI; VGA; DVI (with converter) |
| DH-VEC1605UH-M60 (4K Encoding Card) | Encoding Format | H.264; H.265 |
| DH-VEC1605UH-M60 (4K Encoding Card) | Encoding Capability | (single card) max. 8-ch 4K (4096 × 2160@24 fps/3840 × 2160@30 fps + 12 × 1080p (1920 × 1080@60 fps) supports encoding 4096 × 2160; 3840 × 2160; 2560 × 1440; 1920 × 1080; 1280 × 720; 704 × 576 |
| DH-VEC1605UH-M60 (4K Encoding Card) | Low Speed Data Port | 1-ch RS485, supports PTZ protocol of main stream Dome |
| DH-VEC1604HF-M60 (Optical Fiber Video) | Optical Fiber | LC |
| DH-VEC1604HF-M60 (Optical Fiber Video) | Audio Input | (single card) 16-ch, D826 port (2.0Vp-p, impedance: 1 KΩ) |
| DH-VEC1604HF-M60 (Optical Fiber Video) | Encoding Format | H.264; MPEG4 |
| DH-VEC1604HF-M60 (Optical Fiber Video) | Encoding Capability | (single card) 16-ch 1080p; UXGA; 720p; D1 |
| DH-VEC1604HF-M60 (Optical Fiber Video) | Low Speed Data Port | 2-ch RS485, supports PTZ protocol of main stream Dome |
| Network Video | Encoding Standard | H.264; MPEG4; ONVIF; PISA |
| Network Video | Transmission Protocol | TCP; UDP; RTP; RTSP; RTCP |
| Network Video | Access Device | IPC; DVR; NVS; NVR |
| Network Video | Video Resolution | 12 MP; 8 MP; 5 MP; 3 MP; 1080p; UXGA; 720p; D1; HD1; 2CIF; CIF; QCIF |
| DH-VDC1805H-M60 (HDMI Port) | Video Output | 18-ch HDMI ports for digital video output, 4K point-to-point output display |
| DH-VDC1805H-M60 (HDMI Port) | Audio Output | 18-ch HDMI ports (bulit-in audio), 2 × DB15 independent audio output |
| DH-VDC1805H-M60 (HDMI Port) | Video Format | H.264; H.265; MPEG4; SVAC; MJPEG |
| DH-VDC1805H-M60 (HDMI Port) | Non-standard Decoding | Yes |
| DH-VDC1805H-M60 (HDMI Port) | Decoding Capability | Support 24-ch 4096 × 2160@25 fps, 24-ch 3840 × 2160@30 fps, 96-ch 1080p@30 fps (H.264, H.265), 192-ch 720p@30 fps, 384-ch D1 decoding 96-ch 1080p H.265 decoding 24-ch 1080p SVAC decoding 72-ch non-standard D1 stream decoding |
| DH-VDC1805H-M60 (HDMI Port) | Decoding Resolution | 12 MP; 8 MP; 5 MP; 3 MP; 1080p; UXGA; 720p; D1 |
| DH-VDC1805H-M60 (HDMI Port) | Video Output Resolution | 4096 × 2160@24 fps; 3840 × 2160@30 fps; 1920 × 1080p@60 fps; 1280 × 1024p@60 fps; 1280 × 720p@60 fps; 1024 × 768p@60 fps |
| DH-VDC1805H-M60 (HDMI Port) | Low Speed Data Port | 2-ch RS485, 4-ch alarm input, 2-ch alarm output |
| DH-VDC1805H-M60 (HDMI Port) | Multi-screen Display | 1/4/6/8/9/16/25/36 split views, custom spilt |
| DH-VDC1505H-M60 (HDMI Port) | Video Output | 15-ch HDMI ports for digital video output, 4K point-to-point output display |
| DH-VDC1505H-M60 (HDMI Port) | Audio Output | 15-ch HDMI ports (bulit-in audio), 2 × DB15 independent audio output |
| DH-VDC1505H-M60 (HDMI Port) | Video Format | H.264; H.265; MPEG4; SVAC; MJPEG |
| DH-VDC1505H-M60 (HDMI Port) | Non-standard Decoding | Yes |
| DH-VDC1505H-M60 (HDMI Port) | Decoding Capability | Support 24-ch 4096 × 2160@25 fps, 20-ch 3840 × 2160@30 fps, 80-ch 1080p@30 fps (H.264, H.265), 160-ch 720p@30 fps, 320-ch D1 decoding 80-ch 1080p H.265 decoding 20-ch 1080p SVAC decoding 60-ch non-standard D1 stream decoding |
| DH-VDC1505H-M60 (HDMI Port) | Decoding Resolution | 12 MP; 8 MP; 5 MP; 3 MP; 1080p; UXGA; 720p; D1 |
| DH-VDC1505H-M60 (HDMI Port) | Video Output Resolution | 4096 × 2160@24 fps; 3840 × 2160@30 fps; 1920 × 1080p@60 fps; 1280 × 1024p@60 fps; 1280 × 720p@60 fps; 1024 × 768p@60 fps |
| DH-VDC1505H-M60 (HDMI Port) | Low Speed Data Port | 2-ch RS485, 4-ch alarm input, 2-ch alarm output |
| DH-VDC1505H-M60 (HDMI Port) | Multi-screen Display | 1/4/6/8/9/16/25/36 split views, custom spilt |
| DH-VDC1205H-M60 (HDMI Port) | Video Output | 12-ch HDMI ports for digital video output, 4K point-to-point output display |
| DH-VDC1205H-M60 (HDMI Port) | Audio Output | 12-ch HDMI ports (bulit-in audio), 2 × DB15 independent audio output |
| DH-VDC1205H-M60 (HDMI Port) | Video Format | H.264; H.265; MPEG4; SVAC; MJPEG |
| DH-VDC1205H-M60 (HDMI Port) | Non-standard Decoding | Yes |
| DH-VDC1205H-M60 (HDMI Port) | Decoding Capability | Support 16-ch 4096 × 2160@25 fps, 16-ch 3840 × 2160@30 fps, 64-ch 1080p@30 fps (H.264, H.265), 128-ch 720p@30 fps, 256-ch D1 decoding 64-ch 1080p H.265 decoding 16-ch 1080p SVAC decoding 48-ch non-standard D1 stream decoding |
| DH-VDC1205H-M60 (HDMI Port) | Decoding Resolution | 12 MP; 8 MP; 5 MP; 3 MP; 1080p; UXGA; 720p; D1 |
| DH-VDC1205H-M60 (HDMI Port) | Video Output Resolution | 4096 × 2160@24 fps; 3840 × 2160@30 fps; 1920 × 1080p@60 fps; 1280 × 1024p@60 fps; 1280 × 720p@60 fps; 1024 × 768p@60 fps |
| DH-VDC1205H-M60 (HDMI Port) | Low Speed Data Port | 2-ch RS485, 4-ch alarm input, 2-ch alarm output |
| DH-VDC1205H-M60 (HDMI Port) | Multi-screen Display | 1/4/6/8/9/16/25/36 split views, custom spilt |
| DH-VDC0905H-M60 (HDMI Port) | Video Output | 9-ch HDMI ports for digital video output, 4K point-to-point output display |
| DH-VDC0905H-M60 (HDMI Port) | Audio Output | 9-ch HDMI ports (bulit-in audio), 1 × DB15 independent audio output |
| DH-VDC0905H-M60 (HDMI Port) | Video Format | H.264; H.265; MPEG4; SVAC; MJPEG |
| DH-VDC0905H-M60 (HDMI Port) | Non-standard Decoding | Yes |
| DH-VDC0905H-M60 (HDMI Port) | Decoding Capability | Support 12-ch 4096 × 2160@25 fps, 12-ch 3840 × 2160@30 fps, 48-ch 1080p@30 fps (H.264, H.265), 96-ch 720p@30 fps, 192-ch D1 decoding 48-ch 1080p H.265 decoding 12-ch 1080p SVAC decoding 36-ch non-standard D1 stream decoding |
| DH-VDC0905H-M60 (HDMI Port) | Decoding Resolution | 12 MP; 8 MP; 5 MP; 3 MP; 1080p; UXGA; 720p; D1 |
| DH-VDC0905H-M60 (HDMI Port) | Video Output Resolution | 4096 × 2160@24 fps; 3840 × 2160@30 fps; 1920 × 1080p@60 fps; 1280 × 1024p@60 fps; 1280 × 720p@60 fps; 1024 × 768p@60 fps |
| DH-VDC0905H-M60 (HDMI Port) | Low Speed Data Port | 2-ch RS485, 4-ch alarm input, 2-ch alarm output |
| DH-VDC0905H-M60 (HDMI Port) | Multi-screen Display | 1/4/6/8/9/16/25/36 split views, custom spilt |
| DH-VDC1204B-M60 (BNC Port) | Decoding Capability | (single card) 128-ch D1/ 32-ch 1080 decoding |
| DH-VDC1204B-M60 (BNC Port) | Video Output Port | 12-ch BNC |
| DH-VDC1204B-M60 (BNC Port) | Audio Output Port | 12-ch BNC |
| DH-VDC1204B-M60 (BNC Port) | Low Speed Data Port | 2-ch RS485, supports PTZ protocol of main stream Dome |
| DH-VDC1204B-M60 (BNC Port) | Multi-screen Display | 1/4/6/8/9 split views |
| DH-VDC0804V-M60 (VGA Port） | Decoding Capability | (single card) 32-ch 1080p/64-ch 720p/128-ch D1 decoding |
| DH-VDC0804V-M60 (VGA Port） | Video Output Port | 8-ch VGA |
| DH-VDC0804V-M60 (VGA Port） | Audio Output Port | DB15, 8-ch line out |
| DH-VDC0804V-M60 (VGA Port） | Low Speed Data Port | 2-ch RS485, supports PTZ protocol of main stream Dome |
| DH-VDC0804V-M60 (VGA Port） | Multi-screen Display | 1/4/6/8/9/16/25/32 split views, custom spilt |
| DH-VDC0804S-M60 (BNC Port) | Decoding Capability | (single card) 32-ch 1080p/64-ch 720p/128-ch D1 decoding |
| DH-VDC0804S-M60 (BNC Port) | Video Output Port | 8-ch BNC, support HD-SDI and 3G-SDI HD video output |
| DH-VDC0804S-M60 (BNC Port) | Audio Output Port | DB15, 8-ch line out |
| DH-VDC0804S-M60 (BNC Port) | Low Speed Data Port | 2-ch RS485, supports PTZ protocol of main stream Dome |
| DH-VDC0804S-M60 (BNC Port) | Multi-screen Display | 1/4/6/8/9/16/25/32 split views, custom spilt |
| Function | Interoperability | ONVIF; General; Dahua Private; Hikvision Private |
| Function | Transmission Protocol | TCP; UDP; RTP; RTSP; RTCP |
| Function | Max System Configuration | Output signal 960-ch 1080p@30 fps |
| Function | Screen Splicing | Max. 180 spilts |
| Function | Spliced Screen Function | Splicing and zoom, video fusion, roaming, windowing, and OSD. |
| Function | Map Mode | Raster, online and offline GIS map and more. |
| Function | Video Play | RTMP protocol. Supports live broadcast through FLV. |
| Function | Auto Registration | Auto registration redirection to achieve dynamic load balancing of the auto registered devices. |
| Function | Video Diagnosis | Built-in video quality diagnosis to analyze the image brightness, color cast, contrast, sharpness, and video loss, and graphically display statistical results. |
| Function | Equipment Management | Supports custom business trees with different structures. Supports adjusting the order of nodes such as channels. |
| Function | Recording Playback | Playback in live view Storage and playback in seconds, which ensures to play back the video one second before the device is disconnected from the network and power. |
| Function | Storage Decoupling | Supports storage decoupling. The crash of business server does not affect the storage server. |
| Function | Supplementary Recording | Supports timely recording. |
| Function | Lock of Recorded Video | Supports video locking and unlocking. Lock important videos to avoid overwriting. |
| Function | Control Mode | With its own operation system, it can be connected to the monitor, mouse, and network keyboard. It also supports web manager operation. |
| Function | Multi-screen Display | 1/4/6/8/9/16/25/36 split Supports M × N custom split, M × N ≤ 36 |
| Function | Audio Compression | By the conditions on matching card |
| Function | Video Compression | H.265; H.264; MJPEG; MPEG4; SVAC; MPEG2 |
| Function | Video Resolution | By the conditions on matching card |
| General | Power Supply | 100–240 VAC, 50–60 Hz, 10/5 A, hot swapping |
| General | Power Redundancy | Dual |
| General | Power Consumption | ≤800 W |
| General | Operating Temperature | –10 °C to +55 °C (+14 °F to +131 °F) |
| General | Operating Humidity | <95% (RH), non-condensing |
| General | Storage Temperature | –10 °C to +55 °C (+14 °F to +131 °F) |
| General | Storage Humidity | <95% (RH), non-condensing |
| General | Product Dimensions | 486.2 mm × 496 mm × 310.3 mm (18.98" × 19.53" × 12.25") (H × W × D) |
| General | Packaging Dimensions | 740 mm × 720 mm × 613 mm (29.13" × 28.35" × 24.13") (H × W × D) |
| General | Gross Weight | 30.5 kg (67.24 lb) |
| General | Net Weight | 30 kg (66.19 lb) |
| General | Series | M60 |
| General | Type | Server |

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION