Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section 28 21 00: Video Surveillance

Section 28 21 11: Analog Cameras

VIDEO MATRIX PLATFORM

- – GENERAL

  - SUMMARY

    - Section Includes

      - Section 28 21 17: Video Surveillance – Surveillance Cameras – Camera Housings

      - Section 28 21 19: Video Surveillance – Surveillance Cameras – Camera Mounts

      - Section 28 27 00: Video Surveillance – Video Surveillance Sensors

    - Related Sections

      - [Section 28 33 15: Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

********** Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

| STANDARD | NA |
| --- | --- |

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - List certifying bodies (CE, FCC, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - INSTALLATION REQUIREMENTS

    - Operating Temperature And Humidity

    - Power Requirements

  - WARRANTY

    - Provide manufacturer’s warranty covering [2] years for replacement and repair of defective equipment. Warranty varies country to country.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

ZHEJIANG DAHUA VISION TECHNOLOGY CO., LTD.

Address: No.1399, Binxing Road, Binjiang District, Hangzhou, P.R. China

Postcode: 310053

Tel: +86-571-87688888 28933188

Email: dhoverseas@dhvisiontech.com

Website: www.dahuasecurity.com

    - Substitutions:

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - VIDEO MATRIX PLATFORM | DHI-M80-S24

| System | Main Processor | 64 high-performance octa-core processor |
| --- | --- | --- |
| System | Operating System | Embedded Linux operating system |
| System | Bus | Dual bus for the network and serial port |
| System | Slot | 27 slots in total. Each of the following items has its own slot: the switching card, main control card, and central control card. But the central control card is reserved. The remaining 24 slots are reserved for audio and video cards. Furthermore, the audio and video cards can be freely inserted into their slots without restrictions |
| System | Cabinet | Single power supply (dual power supply optional); single main control board (dual main control board optional); intelligent temperature-controlled fan |
| System | Touch Screen | An 8-inch touch screen with a resolution of 1280 × 800 that enables the visualization of any touch operation for the device |
| Port | USB | 1 × USB 3.0 port and 1 × USB 2.0 port |
| Port | Network Port | 4 × 2.5 Gbps RJ45 port and 2 × 10 Gbps optical port |
| Port | RS-232 | 2 |
| Port | RS-485 | 2 |
| Port | HDMI | 1 (reserved) |
| Port | Alarm Input Port | 2 (reserved) |
| Port | Alarm Output Port | 2 (reserved) |
| Port | Audio Input Port | One 3.5 mm jack port (reserved) |
| Port | Audio Output Port | One 3.5 mm jack port |
| Port | Type-C Port | 1 Type-C port that supports USB 2.0 |
| VEC0205EHP-M80 | Video Input | 2-channel HDMI 2.0 port and 2-channel DP 1.2 port. Either one can be set for each input channel |
| VEC0205EHP-M80 | Audio Input | 2-channel HDMI/DP embedded audio port and 2-channel 3.5 mm jack port |
| VEC0205EHP-M80 | Encoding Format | H.265 |
| VEC0205EHP-M80 | Encoding Capability | Supports 2-ch YUV420 4096 × 2160@60 Hz or 2-ch YUV444 4096 × 2160@30 Hz, with downward resolution compatibility; Supports custom resolutions, with a range of width 800–8192, height 600–8192, and a total pixel width × height ≤ 8847360 (4096 × 2160) |
| VEC0205EHP-M80 | Audio Encoding Format | PCM; G711.A |
| VEC0405HH-M80 | Video Input | 4 × HDMI 1.3 ports |
| VEC0405HH-M80 | Audio Input | 4-channel HDMI embedded audio port and 4-channel 3.5 mm jack port |
| VEC0405HH-M80 | Encoding Format | H.265 |
| VEC0405HH-M80 | Encoding Capability | Supports 4- channel YUV444/YUV420 at 1920 × 1200@60 Hz, with compatibility for lower resolutions. Supports custom resolutions within the range: width 800-3840, height 600-2880, with total pixel count (width × height) ≤ 2304000 (1920 × 1200) |
| VEC0405HH-M80 | Audio Encoding Format | PCM; G711.A |
| VDC0405EH | Video Output (LCD) | 4 × HDMI 2.0 ports. If a layer spans multiple decoding cards, each card must decode the layer individually; 1. When the number of 4K input video walls is less than or equal to 2, it can decode videos at the following resolutions: • 4 channels at 1920 × 1080@60 Hz; • 4 channels at 3840 × 2160@60 Hz. The video will not cross the edge of the LCD screen; • 2 channels at 3840 × 2160@60 Hz. This is for the scene where videos from multiple channels will cross the screen; 2. When the number of 4K input video walls exceeds 2, it can decode videos at the following resolutions: • 4 channels at 1920 × 1080@60 Hz; • 4 channels at 3840 × 2160@60 Hz. The signal will not cross the edge of the video screen; • 1 channel at 3840 × 2160@60 Hz. This is for the scene where videos from multiple channels will cross the screen |
| VDC0405EH | Video Output (LED) | 4 HDMI 2.0 ports. The layer covers multiple decoding cards, each requiring one decoding operation; Supports embedded audio output; Supports 2-ch 4K or 4-ch 1080p; Supports custom resolutions, four output ports share the same total output capacity: 1) To ensure output port synchronization, the resolution of each individual card and the entire video wall must be consistent; 2) Width range: 200–3840, height range: 200–3840; 3) Maximum total pixels per port: width × height ≤ 8294400 (3840 × 2160); 4) Total pixel load capacity of each board's output ports ≤ 17MP |
| VDC0405EH | Video Format | Supports decoding of video compression formats: H.265; H.264; MJPEG; MPEG4; SVAC (customizable); and MPEG2 |
| VDC0405EH | Decoding Capability | Supports performing real-time decoding of network videos based on the following capabilities: 2-ch 32MP@25 fps; 4-ch 8MP@60 fps; 8-ch 8MP@30 fps; 36-ch 1080p@30 fps; 18-ch 1080p@60 fps and 130-ch D1@25 fps. Only H.265 can be used to decode videos in 32 MP |
| VDC0405EH | Decoding Resolution | 32 MP, 16 MP, 12 MP, 8 MP, 5 MP, 3 MP, 1080p, UXGA, 720p, D1, HD1, 2CIF, CIF, QCIF. 32 MP and 16 MP are supported only in H.265 |
| VDC0405EH | Video Output Resolution | 3840 × 2160@60 Hz, 3840 × 2160@30 Hz, 2048 × 1152@60 Hz, 1920 × 1200@60 Hz, 1920 × 1080@60 Hz, 1280 × 1024@60 Hz, 1280 × 720@60 Hz, 1024 × 768@60 Hz. Supports custom resolution output (e.g., 2160 × 3840@60 Hz, 3072 × 2700@60 Hz) |
| VDC0405EH | Multi-screen Display | Supports 1/4/6/8/9/16/25/36 screen split, and supports M × N custom split. Each output port supports up to 36 screens (others customizable); a single board supports up to 144 screens; the entire unit supports up to 3456 screens |
| Function | Interoperability | ONVIF; RTSP; Hikvision Private; Dahua Private |
| Function | Transmission Protocol | TCP; UDP; RTP; RTSP; RTCP |
| Function | Max System Configuration | Supports installation of up to 24 audio and video cards. The maximum output resolution is 96-ch 4K@60 Hz. The maximum input resolution is 96-ch 1080p@60 Hz or 48-ch 4K@60 Hz |
| Function | Network Signal Access | Up to 10,000 network channels can be accessed. If you want to adjust the number of network channels to be accessed, please contact the manufacturer |
| Function | Screen Splicing | Splices up to 96 screens |
| Function | Spliced Screen Function | Splicing and zoom, video integration, roaming, windowing, and OSD |
| Function | Video Wall | Supports 16 video walls |
| Function | Subtitle | Single video wall: a total of 8 captions, including 1 scrolling caption and 7 static captions |
| Function | Scene | 1. Supports video scenes; supports up to 128 scenes; 2. Supports additional presets (with iSee): supports up to 128 presets; 3. Supports combined presets (with iSee): supports up to 128 presets |
| Function | Scenario Tour | 1. Supports scene tour, with tour interval adjustable from 10 to 300 seconds; 2. Supports scheduled switching, with settings configurable by day of the week, and multiple time periods can be set per day |
| Function | Background Capability | 1. Number of background images: 64; 2. Background image resolution: width × height × 3 < 400 × 1024 × 1024; 3. Background image types: jpg, bmp |
| Function | Output Setting | 1. Supports configuration of screen chroma, brightness, contrast, and saturation; 2. Output layer formats supported: YUV422, XRGB8888; 3. Audio output modes: 3.5mm, HDMI, HDMI & 3.5mm; default is 3.5mm |
| Function | Smart Display | 1. Supports smart rules; 2. Supports intelligent tracking frames; 3. Supports metadata of face, people, vehicle, non-motor vehicle, and crowd density |
| Function | Fan Control | 1. 8 smart fans; 2. Supports normal mode and conference room mode; 3. Supports fan group configuration, with temperature ranges corresponding to low, medium, and high speeds |
| Function | Equipment Management | 1. Supports local signals and network signals; 2. Supports custom signal grouping (5-level grouping) and favorites (2-level grouping) |
| Function | SDK | Supports secondary development, and provides platform SDK |
| Function | Control Mode | Operations can be performed directly through the device's touch screen. You can also connect to the device through the webpage, desktop client, mobile client and network keyboard |
| Function | Multi-screen Display | Splits to 1/4/6/8/9/16/25/36 windows, and supports M × N custom split |
| General | Power Supply | 100–127 VAC/100–240 VAC, 50–60 Hz, 10/5 A, hot swapping |
| General | Power Redundancy | Dual |
| General | Power Consumption | ≤1200 W |
| General | Operating Temperature | –10 °C to +50 °C (+14 °F to +122 °F) |
| General | Operating Humidity | 10%–90% (non-condensing) |
| General | Storage Temperature | –10 °C to +50 °C (+14 °F to +122 °F) |
| General | Storage Humidity | 10%–90% (non-condensing) |
| General | Product Dimensions | 354.8 mm × 484.0 mm × 409.2 mm (13.94" × 19.06" × 16.11") (H × W × D) |
| General | Packaging Dimensions | 554 mm × 622 mm × 563 mm (21.81" × 24.49" × 22.13") (H × W × D) |
| General | Gross Weight | 28 kg (61.73 lb) |
| General | Net Weight | 23 kg (50.71 lb) |
| General | Series | M80 |
| General | Type | Server |

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION